# ECE 444 - Digital Signal Processing

# Homework 2

Cole Zenk

## Problem 1: Book problem 2.1-5(d)

Determine and plot the poles and zeros of H(s), use the pole and zero information to predict overall system behavior. Confirm your predictions by graphing the system’s frequency response (magnitude and phase).

Given transfer function:

H(s)=0.94s4+11.33s2+33.99s4+0.94s3+12.66s2+5.64s+36

To get anything out of this for intuition, the transfer function must be factored. Factoring a 4th order polynomial by hand is impractical when tools like MATLAB or Python are readily available.

>>> import numpy as np
>>> num = [.94, 0, 11.33, 0, 33.99]
>>> den = [1, .94, 12.66, 5.64, 36]
>>> zeros = np.roots(num)
>>> poles = np.roots(den)
>>> print(zeros)
[-2.22e-16+2.535j -2.22e-16-2.535j  2.22e-16+2.372j  2.22e-16-2.372j]
>>> print(poles)
[-0.267+2.792j -0.267-2.792j -0.203+2.129j -0.203-2.129j]

Factored transfer function:

H(s)=(s±2.535j)2(s±2.372j)2(s+0.267±2.792j)2(s+0.203±2.129j)2

This transfer function shows that there will be suppression near 2.372 and 2.535 rad/s, and amplification near 2.792 and 2.129 rad/s. The following plots confirm this.

Simulation:

>>> import dsp_sim_utils as now
>>> now.plot_phase_mag(num, den)
>>> now.plot_pole_zero(num, den, 'Pole Zero Question 1')

## Problem 2: Book problem 2.2-2

Consider the all-pass filter specified by the transfer function:

H(s)=s−2s+2

Verify the all-pass character of the filter by plotting the system frequency response (magnitude and phase). Can this filter be considered distortionless? Explain.

Simulation:

>>> now.plot_phase_mag([1, -2], [1, 2], 'Question 2 plots')

All-Pass Filter

Analysis: The magnitude plot shows constant 0 dB across all frequencies, confirming all-pass behavior. However, the phase is nonlinear (varies with frequency), so this filter is not distortionless. Distortionless transmission requires both constant magnitude AND linear phase.

## Problem 3: Book problem 2.3-3

Circuit: Series R (1 kΩ) with parallel C (1 nF) to ground, forming a lowpass RC filter.

### Part 1: Find the system transfer function

Using voltage division, the circuit can be analyzed as:

y(t)=x(t)×ZCZC+ZR

Y(s)=X(s)×1sCR+1sC

H(s)=1sRC+1

With values R = 1 kΩ, C = 1 nF:

H(s)=1s×10−6+1

Cutoff frequency: ωc=1RC=106 rad/s (1 MHz)

### Part 2: Plot the magnitude and phase responses

>>> now.plot_phase_mag([1], [1e-6, 1], 'Question 3 plots', ylim_mag=[-1, 1])

RC Filter Response

### Part 3: Show distortionless transmission for lowpass signals

Magnitude analysis:

H(jω)=1jω×10−6+1

|H(jω)|=1(ω×10−6)2+1

For ω≪106 rad/s: |H(jω)|≈1 (flat passband) ✓

Phase analysis:

∠H(jω)=−tan−1(ω×10−6)

By Taylor expansion, for x≪1:

arctan(x)=n=0∞(−1)n2n+1x2n+1≈x

For ω≪106:

∠H(jω)≈−ω×10−6

This is linear phase of the form ϕ(ω)=−ωτ where τ=RC=10−6 seconds.

In the passband:

H(jω)≈e−jω×10−6

Y(jω)=X(jω)⋅e−jω×10−6

By the Fourier transform shifting property:

y(t)=x(t−10−6)

Conclusion: For any lowpass signal with ω≪106 rad/s, the output experiences negligible distortion with only a 1 microsecond time delay.

### Part 4: Bandpass signal with carrier

For x(t)=g(t)cos(ωct) where ωc=3×106 rad/s and g(t) has 50 Hz bandwidth:

The carrier frequency (3 MHz) is well above the cutoff (1 MHz), experiencing significant attenuation (~10 dB). However, the slowly-varying envelope g(t) (50 Hz) passes through relatively undistorted. The output is approximately:

y(t)≈K⋅g(t−10−6)cos(3×106t−ϕ)

where K≈0.3 is the attenuation factor at 3 MHz and ϕ is the phase shift.

## Problem 4: Book problem 2.4-3

For each of the following windows, determine a suitable width T so that when applied to an ideal lowpass filter, the resulting transition band is approximately 1 rad/s.

Method: Set main lobe width = 1 rad/s and solve for T.

| Window | Main Lobe Width | Equation | T (seconds) |
| --- | --- | --- | --- |
| Rectangular | 4πT | 4πT=1 | 12.57 |
| Triangular | 8πT | 8πT=1 | 25.13 |
| Hann | 8πT | 8πT=1 | 25.13 |
| Hamming | 8πT | 8πT=1 | 25.13 |
| Blackman | 12πT | 12πT=1 | 37.70 |

## Problem 5: Book problem 2.5-1

Consider a microphone intended for use in a music recording studio. Determine a suitable frequency response (lowpass, highpass, bandpass, or bandstop) for the microphone, and provide suitable values (such as δp and ωs) to specify the response.

Filter type: Bandpass

Rationale: A music recording studio microphone should capture the full range of musical instruments and human hearing while rejecting out-of-band noise.

Specifications:

- Passband edges: ωp1=20 Hz (125.7 rad/s), ωp2=20 kHz (125,664 rad/s)

- Stopband edges: ωs1=10 Hz (62.8 rad/s), ωs2=25 kHz (157,080 rad/s)

- Passband ripple: δp=0.05 → αp=0.45 dB

- Stopband ripple: δs=0.01 → αs=40 dB

## Problem 6: Book problem 2.6-1(b,c)

Determine the filter H(s) that results by applying the following transformations on a lowpass prototype filter:

Hp(s)=1s3+2s2+2s+1

Plot the corresponding magnitude response. In each case, designate ω0 as the prototype’s 1-dB cutoff frequency.

### Finding ω0

Using numerical methods:

>>> omega_0 = find_omega_0([1], [1, 2, 2, 1], cutoff_db=-1)
>>> print(omega_0)
0.798 rad/s

Prototype Lowpass

### Part (b): Lowpass-to-Highpass transformation with ω1=5 rad/s

Transformation: s→ω0ω1s=3.99s

Result:

HHP(s)=s3s3+7.98s2+31.84s+63.52

>>> k = omega_0 * omega_1
>>> num_hp = [1, 0, 0, 0]
>>> den_hp = [1, 2*k, 2*k**2, k**3]
>>> now.plot_phase_mag(num_hp, den_hp, 'HP Plot Question 6')

Highpass Filter

### Part (c): Lowpass-to-Bandpass transformation with ω1=3 rad/s, ω2=4 rad/s

Transformation: s→s2+ω1ω2s(ω2−ω1)=s2+12s

Result (from symbolic computation):

HBP(s)=s3s6+2s5+38s4+49s3+456s2+288s+1728

>>> num_bp = [1, 0, 0, 0]
>>> den_bp = [1, 2, 38, 49, 456, 288, 1728]
>>> now.plot_phase_mag(num_bp, den_bp, 'BP Plot Question 6')

Bandpass Filter

Note: The bandpass filter is 6th order (double the prototype order), which is expected for LP→BP transformations.

## Appendix: Python Utilities

All simulations used the dsp_sim_utils.py module:

# ECE 444 - Digital Signal Processing
# Homework 2
**Cole Zenk**

---

## Problem 1: Book problem 2.1-5(d)

Determine and plot the poles and zeros of H(s), use the pole and zero information to predict overall system behavior. Confirm your predictions by graphing the system's frequency response (magnitude and phase).

**Given transfer function:**

$$
H(s) = \frac{0.94s^{4} + 11.33s^{2} + 33.99}{s^{4} + 0.94s^{3} + 12.66s^{2} + 5.64s + 36}
$$

To get anything out of this for intuition, the transfer function must be factored. Factoring a 4th order polynomial by hand is impractical when tools like MATLAB or Python are readily available.

```python
>>> import numpy as np
>>> num = [.94, 0, 11.33, 0, 33.99]
>>> den = [1, .94, 12.66, 5.64, 36]
>>> zeros = np.roots(num)
>>> poles = np.roots(den)
>>> print(zeros)
[-2.22e-16+2.535j -2.22e-16-2.535j  2.22e-16+2.372j  2.22e-16-2.372j]
>>> print(poles)
[-0.267+2.792j -0.267-2.792j -0.203+2.129j -0.203-2.129j]

Factored transfer function:

H(s)=(s±2.535j)2(s±2.372j)2(s+0.267±2.792j)2(s+0.203±2.129j)2

This transfer function shows that there will be suppression near 2.372 and 2.535 rad/s, and amplification near 2.792 and 2.129 rad/s. The following plots confirm this.

Simulation:

>>> import dsp_sim_utils as now
>>> now.plot_phase_mag(num, den)
>>> now.plot_pole_zero(num, den, 'Pole Zero Question 1')

## Problem 2: Book problem 2.2-2

Consider the all-pass filter specified by the transfer function:

H(s)=s−2s+2

Verify the all-pass character of the filter by plotting the system frequency response (magnitude and phase). Can this filter be considered distortionless? Explain.

Simulation:

>>> now.plot_phase_mag([1, -2], [1, 2], 'Question 2 plots')

All-Pass Filter

Analysis: The magnitude plot shows constant 0 dB across all frequencies, confirming all-pass behavior. However, the phase is nonlinear (varies with frequency), so this filter is not distortionless. Distortionless transmission requires both constant magnitude AND linear phase.

## Problem 3: Book problem 2.3-3

Circuit: Series R (1 kΩ) with parallel C (1 nF) to ground, forming a lowpass RC filter.

### Part 1: Find the system transfer function

Using voltage division, the circuit can be analyzed as:

y(t)=x(t)×ZCZC+ZR

Y(s)=X(s)×1sCR+1sC

H(s)=1sRC+1

With values R = 1 kΩ, C = 1 nF:

H(s)=1s×10−6+1

Cutoff frequency: ωc=1RC=106 rad/s (1 MHz)

### Part 2: Plot the magnitude and phase responses

>>> now.plot_phase_mag([1], [1e-6, 1], 'Question 3 plots', ylim_mag=[-1, 1])

RC Filter Response

### Part 3: Show distortionless transmission for lowpass signals

Magnitude analysis:

H(jω)=1jω×10−6+1

|H(jω)|=1(ω×10−6)2+1

For ω≪106 rad/s: |H(jω)|≈1 (flat passband) ✓

Phase analysis:

∠H(jω)=−tan−1(ω×10−6)

By Taylor expansion, for x≪1:

arctan(x)=n=0∞(−1)n2n+1x2n+1≈x

For ω≪106:

∠H(jω)≈−ω×10−6

This is linear phase of the form ϕ(ω)=−ωτ where τ=RC=10−6 seconds.

In the passband:

H(jω)≈e−jω×10−6

Y(jω)=X(jω)⋅e−jω×10−6

By the Fourier transform shifting property:

y(t)=x(t−10−6)

Conclusion: For any lowpass signal with ω≪106 rad/s, the output experiences negligible distortion with only a 1 microsecond time delay.

### Part 4: Bandpass signal with carrier

For x(t)=g(t)cos(ωct) where ωc=3×106 rad/s and g(t) has 50 Hz bandwidth:

The carrier frequency (3 MHz) is well above the cutoff (1 MHz), experiencing significant attenuation (~10 dB). However, the slowly-varying envelope g(t) (50 Hz) passes through relatively undistorted. The output is approximately:

y(t)≈K⋅g(t−10−6)cos(3×106t−ϕ)

where K≈0.3 is the attenuation factor at 3 MHz and ϕ is the phase shift.

## Problem 4: Book problem 2.4-3

For each of the following windows, determine a suitable width T so that when applied to an ideal lowpass filter, the resulting transition band is approximately 1 rad/s.

Method: Set main lobe width = 1 rad/s and solve for T.

| Window | Main Lobe Width | Equation | T (seconds) |
| --- | --- | --- | --- |
| Rectangular | 4πT | 4πT=1 | 12.57 |
| Triangular | 8πT | 8πT=1 | 25.13 |
| Hann | 8πT | 8πT=1 | 25.13 |
| Hamming | 8πT | 8πT=1 | 25.13 |
| Blackman | 12πT | 12πT=1 | 37.70 |

## Problem 5: Book problem 2.5-1

Consider a microphone intended for use in a music recording studio. Determine a suitable frequency response (lowpass, highpass, bandpass, or bandstop) for the microphone, and provide suitable values (such as δp and ωs) to specify the response.

Filter type: Bandpass

Rationale: A music recording studio microphone should capture the full range of musical instruments and human hearing while rejecting out-of-band noise.

Specifications:

- Passband edges: ωp1=20 Hz (125.7 rad/s), ωp2=20 kHz (125,664 rad/s)

- Stopband edges: ωs1=10 Hz (62.8 rad/s), ωs2=25 kHz (157,080 rad/s)

- Passband ripple: δp=0.05 → αp=0.45 dB

- Stopband ripple: δs=0.01 → αs=40 dB

## Problem 6: Book problem 2.6-1(b,c)

Determine the filter H(s) that results by applying the following transformations on a lowpass prototype filter:

Hp(s)=1s3+2s2+2s+1

Plot the corresponding magnitude response. In each case, designate ω0 as the prototype’s 1-dB cutoff frequency.

### Finding ω0

Using numerical methods:

>>> omega_0 = find_omega_0([1], [1, 2, 2, 1], cutoff_db=-1)
>>> print(omega_0)
0.798 rad/s

Prototype Lowpass

### Part (b): Lowpass-to-Highpass transformation with ω1=5 rad/s

Transformation: s→ω0ω1s=3.99s

Result:

HHP(s)=s3s3+7.98s2+31.84s+63.52

>>> k = omega_0 * omega_1
>>> num_hp = [1, 0, 0, 0]
>>> den_hp = [1, 2*k, 2*k**2, k**3]
>>> now.plot_phase_mag(num_hp, den_hp, 'HP Plot Question 6')

Highpass Filter

### Part (c): Lowpass-to-Bandpass transformation with ω1=3 rad/s, ω2=4 rad/s

Transformation: s→s2+ω1ω2s(ω2−ω1)=s2+12s

Result (from symbolic computation):

HBP(s)=s3s6+2s5+38s4+49s3+456s2+288s+1728

>>> num_bp = [1, 0, 0, 0]
>>> den_bp = [1, 2, 38, 49, 456, 288, 1728]
>>> now.plot_phase_mag(num_bp, den_bp, 'BP Plot Question 6')

Bandpass Filter

Note: The bandpass filter is 6th order (double the prototype order), which is expected for LP→BP transformations.

## Problem 7: Book problem 2.6-3(a,c,d)

[Not completed]

## Appendix: Python Utilities

All simulations used the dsp_sim_utils.py module:

import matplotlib.pyplot as plot
from scipy import signal
import numpy as np

def plot_phase_mag(num, den, title=None, save_path=None, ylim_mag=None):
    """Plot Bode magnitude and phase response"""
    # Implementation in dsp_sim_utils.py

def plot_pole_zero(num, den, title=None, save_path=None):
    """Plot pole-zero diagram"""
    # Implementation in dsp_sim_utils.py

def find_omega_0(num, den, cutoff_db=-1):
    """Find cutoff frequency for a given dB level"""
    # Implementation in dsp_sim_utils.py

```